[image: image1.png]


[image: image2.png]



MI C RO - I N VE R T E R
 The first commercially available Micro-Inverter
System for residential and commercial solar PV applications. The Micro-Inverter System utilizes advanced technologies to maximize energy harvest, increase system reliability and dramatically simplify design, installation and management.
PR ODU C TIV E
R E L I A B L E
S M A R T

- Maximum energy production
- Resilient to dust, debris and shading
- Performance monitoring per module
- MTBF of 119 years
- System availability greater than 99.8%
- No single point of system failure
- Quick & simple design, installation and management
- 24/7 monitoring and analysis
MICRO-INVERTER TECHNICAL DATA
	24V Modules
32V Modules

	208V
240V
208V
240V

	Input Data (DC)
M175-24-208-S01/2
M175-24-240-S01/2    M200-32-208-S01/2
M200-32-240-S01/2

	Recommended input power (STC)
210W
210W
240W
240W
Maximum input DC voltage
54V
54V
80V
80V
Peak power tracking voltage
25V – 40V
25V – 40V
44V – 65V
44V – 65V
Max. DC short circuit current
10A
10A
7A
7A
Max. input current
8A
8A
5A
5A

	Output Data (AC)

	Maximum output power
175W
175W
200W
200W
Nominal output current
840mA
750mA
960mA
833mA
Nominal voltage/range
208V/183V-229V   240V/211V-264V   208V/183V-229V
240V/211V-264V
Extended voltage/range
208V/179V-232V   240V/206V-269V   208V/179V-232V
240V/206V-269V
Nominal frequency/range
60.0/59.3-60.5
60.0/59.3-60.5
60.0/59.3-60.5
60.0/59.3-60.5
Extended frequency/range
60.0/59.2-60.6
60.0/59.2-60.6
60.0/59.2-60.6
60.0/59.2-60.6
Power factor
>0.95
>0.95
>0.95
>0.95
Maximum units per branch
24
16
21
14

	Efficiency

	Peak inverter efficiency
95%
95%
95.3%
95.3%
CEC weighted efficiency
94.5%
94.5%
95.0%
95.0%
Nominal MPP tracking
99.6%
99.6%
99.8%
99.8%

	Mechanical Data

	Operating temperature range
-25ºC to +65ºC
-25ºC to +65ºC
-25ºC to +65ºC
-25ºC to +65ºC
Night time power consumption
560mW
580mW
438mW
464mW
Dimensions (WxHxD)
10.5” x 5.5” x 1.5”
Weight
4.0 lbs
Cooling
Natural Convection – No Fans
Enclosure environmental rating
Outdoor – NEMA 6

	Features

	Communication
Powerline
Warranty
15 Years
Compliance
UL1741/IEEE1547
FCC Part 15 Class B



Printed on 100 percent recycled paper.
�








�











